# Ego-Centric Collision Anticipation - Label Schema and Benchmark Specification

# 1. Purpose

This document defines a label schema and benchmark specification for an enterprise-grade ego-centric collision anticipation system using vehicle-mounted front-camera video.

It is designed so that the benchmark can be applied later to:

- customer-provided videos,

- public dashcam datasets,

- internal replay corpora,

- near-miss and risky-event mining pipelines,

- online advisory / warning evaluation.

The benchmark is intentionally ego-centric:

- positive events should be defined with respect to the ego vehicle,

- non-ego accidents should not be treated as equivalent positives,

- hazard localization and warning timing are treated as first-class outputs.

# 2. Benchmark Scope

## 2.1 Primary task

Predict whether the ego vehicle will collide or near-collide within future horizon (H), using front-camera egocentric video.

## 2.2 Secondary tasks

- identify the primary hazard actor,

- estimate time-to-event,

- classify impact sector,

- produce explanation / reason code,

- localize the relevant collision / hazard region.

## 2.3 Deployment modes supported

- offline event mining,

- offline benchmark evaluation,

- real-time advisory / warning evaluation,

- fleet safety analytics.

# 3. Label Ontology

# 3.1 Event-level labels

Every benchmark clip or event window should receive one of the following primary labels:

## A. collision

The ego vehicle is involved in physical impact with another actor or object.

## B. near_collision

The ego vehicle enters a critical conflict state where collision is narrowly avoided or would be highly likely without evasive action.

## C. normal

No critical ego-involved collision or near-collision occurs within the benchmark horizon.

## D. ambiguous

Optional label used only for annotation review and exclusion. Use when event relevance is unclear or visibility is too poor for reliable ground truth.

# 3.2 Ego relevance labels

To prevent dataset leakage from irrelevant accidents, every event candidate should also be labeled for ego relevance.

## ego_involved

- yes

- no

- uncertain

### Rule

Only clips with ego_involved = yes should count as positive for the primary collision-anticipation benchmark.

# 3.3 Event severity labels

Optional but recommended.

## severity

- collision_minor

- collision_moderate

- collision_severe

- near_collision_low

- near_collision_high

Severity definitions should be customer- and business-aligned.

# 3.4 Scenario type labels

Each event or clip should receive one or more scenario labels.

## Suggested taxonomy

- rear_end

- cut_in_left

- cut_in_right

- cross_traffic

- side_swipe_left

- side_swipe_right

- stopped_vehicle_ahead

- pedestrian_intrusion

- cyclist_conflict

- motorcycle_conflict

- intersection_conflict

- merging_conflict

- lane_departure_conflict

- obstacle_in_lane

- multi_vehicle_chain

- unknown_conflict

# 4. Temporal Label Schema

Temporal supervision is essential for anticipation benchmarks.

# 4.1 Clip-level temporal fields

## clip_start_ts

Timestamp of clip start within trip or source video.

## clip_end_ts

Timestamp of clip end.

## fps_nominal

Nominal video frame rate.

# 4.2 Event timing fields

## event_time

Timestamp of actual collision or near-collision moment.

## event_start

Timestamp when the dangerous event sequence begins.

## alert_time

Earliest timestamp where a trained human annotator can reasonably identify the event as predictable.

## first_visible_hazard_time

Timestamp when the hazard actor first becomes visible.

## hazard_commitment_time

Optional label for when the event becomes strongly committed or difficult to avoid.

# 4.3 Derived timing labels

These can be computed from the above.

## tta_ms

Time-to-accident / time-to-event at each frame or clip endpoint.

## predictable_horizon_ms

event_time - alert_time

## visibility_horizon_ms

event_time - first_visible_hazard_time

# 5. Spatial Label Schema

# 5.1 Hazard actor labels

Each positive event should identify the primary hazard actor.

## hazard_actor_id

Unique actor identifier for the main risk source.

## hazard_actor_class

- car

- truck

- bus

- motorcycle

- bicycle

- pedestrian

- barrier

- static_object

- animal

- unknown

# 5.2 Bounding box / track labels

For hazard-aware benchmarks, annotate:

## hazard_bbox

Bounding box of primary hazard actor at key timestamps.

## hazard_track

Track of hazard actor across the event window.

## secondary_actor_tracks

Optional tracks for other involved actors.

# 5.3 Impact localization labels

## impact_sector

- front_left

- front_center

- front_right

- left_side

- right_side

- rear_like

- unknown

## impact_region_bbox

Optional image-space region where collision is expected or occurs.

## collision_point_visible

- yes

- no

- partially_visible

# 6. Explanation and Cause Labels

These labels make the benchmark more useful for enterprise products.

## reason_code

Primary cause or risk explanation.

Suggested taxonomy:

- lead_vehicle_brake

- rapid_closing_speed

- cut_in_left

- cut_in_right

- cross_traffic_entry

- unsafe_merge

- stopped_vehicle_ahead

- pedestrian_step_in

- cyclist_intrusion

- ego_lane_departure

- occluded_actor_emergence

- multi_actor_conflict

- unknown_reason

## preventive_action

Optional.

- brake_earlier

- increase_headway

- yield

- maintain_lane

- reduce_speed

- improve_gap_selection

- no_clear_prevention

# 7. Context Labels

These support robust benchmarking and condition-wise analysis.

## Lighting

- day

- night

- dawn_dusk

- tunnel_transition

## Weather

- clear

- rain

- fog

- snow

- wet_road

- unknown

## Road type

- urban

- highway

- rural

- intersection

- parking_lot

- merge_ramp

## Traffic density

- low

- medium

- high

## Visibility quality

- good

- moderate

- poor

## Camera condition

- clean

- glare

- dirty_windshield

- blur

- shake

- occluded

# 8. Driver-Behavior Extension Labels

If the benchmark is later extended to behavior analysis, attach optional behavior tags.

## Example tags

- tailgating

- aggressive_lane_change

- unsafe_overtake

- poor_gap_acceptance

- weaving

- late_braking

- defensive_driving

- normal_behavior

These should not replace collision labels; they should complement them.

# 9. Annotation Unit Definitions

## 9.1 Trip

Continuous driving session.

## 9.2 Clip

A contiguous window extracted from a trip, for example 10–40 seconds.

## 9.3 Event window

The temporal segment centered around a positive event or candidate risk event.

## 9.4 Frame annotation keypoints

Certain frames should receive full actor / spatial annotations:

- first visible hazard frame,

- alert time frame,

- event time frame,

- peak risk frame,

- optional intermediate frames every N frames.

# 10. Benchmark Tasks

# 10.1 Task A — Binary ego-centric risk prediction

## Input

Clip or streaming window.

## Output

p(collision_or_near_collision within H)

## Horizon set

Recommended:

- 500 ms

- 1000 ms

- 1500 ms

- optional 2000 ms and 3000 ms

# 10.2 Task B — Multi-horizon anticipation

## Input

Clip ending at time (t)

## Output

Multiple probabilities:

- p_event_500ms

- p_event_1000ms

- p_event_1500ms

- p_event_2000ms

This avoids collapsing anticipation into a single threshold.

# 10.3 Task C — Hazard actor localization

## Input

Clip or frame sequence.

## Output

- primary hazard track ID,

- hazard bounding box,

- optional ranked actor list.

# 10.4 Task D — Impact sector prediction

## Input

Clip or final observation window.

## Output

Predicted impact_sector.

# 10.5 Task E — Reason code prediction

## Input

Clip or fused scene graph.

## Output

Predicted reason_code.

# 10.6 Task F — Time-to-event regression

## Input

Clip or frame sequence.

## Output

Estimated tta_ms or bucketed horizon class.

# 11. Benchmark Splits

# 11.1 Required split structure

Use trip- and vehicle-aware splits to prevent leakage.

## Split recommendation

- train

- validation

- test_in_domain

- test_out_of_domain

# 11.2 Leakage prevention rules

Never allow:

- the same trip in multiple splits,

- temporally adjacent clips from the same event in both train and test,

- near-identical duplicates across splits,

- the same vehicle-camera setup dominating all splits without balanced representation.

# 11.3 Recommended domain-based test partitions

Create condition slices such as:

- day vs night,

- clear vs rain,

- urban vs highway,

- high-speed vs low-speed,

- high traffic vs sparse traffic,

- different camera vendors / mounting angles.

# 12. Sampling Strategy for Benchmark Construction

# 12.1 Positive sampling

For each positive event, create multiple clips ending at different offsets before the event.

Example offsets:

- 0.5 s before event,

- 1.0 s before event,

- 1.5 s before event,

- 2.0 s before event,

- 3.0 s before event.

# 12.2 Negative sampling

Negatives must include more than easy normal driving.

Include:

- ordinary driving,

- aggressive but safe situations,

- strong motion but no conflict,

- non-ego accidents,

- shoulder obstacles,

- parked vehicles,

- adjacent-lane events not threatening ego.

Hard negatives are essential.

# 12.3 Near-collision sampling

Near-collision events should be sampled similarly to collisions, but clearly labeled and separable if needed.

Recommended options:

- treat as positive in main benchmark,

- and also report collision-only subset performance.

# 13. Metrics Specification

# 13.1 Primary metrics

## A. AP by anticipation horizon

Compute average precision separately at:

- 500 ms,

- 1000 ms,

- 1500 ms,

- optional longer horizons.

## B. AUC

Report ROC AUC for overall event prediction.

## C. Mean time-to-accident (mTTA)

Average lead time at which events are correctly anticipated.

# 13.2 Secondary metrics

## Hazard localization

- top-1 hazard actor accuracy,

- box IoU at alert frame,

- track localization accuracy.

## Impact prediction

- impact sector accuracy.

## Explanation

- reason-code accuracy,

- macro-F1 over reason codes.

## Regression

- MAE / RMSE on TTA.

# 13.3 Product metrics

For enterprise deployment, include:

- false alerts per driving hour,

- missed critical events per 1000 km,

- median / P95 alert lead time,

- nuisance alert rate by condition,

- severity-weighted miss rate,

- event review usefulness.

# 13.4 Condition-wise reporting

Always report metrics by:

- day/night,

- weather,

- road type,

- traffic density,

- object class,

- visibility quality,

- speed regime if available.

# 14. Acceptance Criteria for Pilot Benchmark

Example acceptance criteria for early pilot:

## Offline event mining

- strong recall on collision and near-collision,

- useful event ranking,

- hazard actor visible in most flagged clips,

- acceptable false positive density for analyst review.

## Online advisory prototype

- warning lead time above business threshold,

- false alerts per hour below defined limit,

- stable hazard localization,

- reason code available for audible alerts.

These values should be finalized with customer stakeholders.

# 15. Annotation Workflow Specification

# 15.1 Recommended annotation stages

## Stage 1 — Event triage

Annotator marks:

- collision / near-collision / normal,

- ego relevance,

- rough event time,

- scenario type.

## Stage 2 — Temporal refinement

Annotator marks:

- event start,

- event time,

- alert time,

- first visible hazard time.

## Stage 3 — Spatial refinement

Annotator marks:

- hazard actor box,

- hazard track,

- impact sector,

- optional region of impact.

## Stage 4 — Context and reason labels

Annotator marks:

- weather,

- lighting,

- road type,

- reason code,

- optional preventive action.

# 15.2 Quality control rules

Use:

- double annotation for positive events,

- adjudication for disagreement,

- consensus for alert time,

- audit sampling for label drift,

- clear annotation guide with examples.

# 16. Suggested JSON Schema

{

"clip_id": "trip123_clip045",

"trip_id": "trip123",

"video_path": "...",

"clip_start_ts": 120.0,

"clip_end_ts": 160.0,

"fps_nominal": 30,

"event_label": "near_collision",

"ego_involved": "yes",

"severity": "near_collision_high",

"scenario_type": ["cut_in_left", "rapid_closing_speed"],

"event_start": 149.2,

"event_time": 151.0,

"alert_time": 149.8,

"first_visible_hazard_time": 147.5,

"hazard_actor_id": 27,

"hazard_actor_class": "car",

"impact_sector": "front_left",

"reason_code": "cut_in_left",

"preventive_action": "reduce_speed",

"context": {

"lighting": "day",

"weather": "clear",

"road_type": "urban",

"traffic_density": "high",

"visibility_quality": "good",

"camera_condition": "clean"

},

"frames": [

{

"frame_index": 900,

"timestamp": 150.0,

"hazard_bbox": [820, 290, 1010, 520],

"hazard_track_visible": true

}

]

}

# 17. Benchmark Leaderboard Template

For each model, report:

- AP@500ms

- AP@1000ms

- AP@1500ms

- AUC

- mTTA

- Hazard Actor Accuracy

- Impact Sector Accuracy

- Reason-Code F1

- False Alerts / Hour

- Latency P50 / P95

- Throughput

If online system:

- edge device type,

- input FPS,

- effective inference FPS,

- power or compute budget if possible.

# 18. Recommended First Version

If customer data is limited, start with a benchmark spec that requires only:

- clip-level event label,

- ego relevance,

- event time,

- hazard actor ID if possible,

- coarse reason code,

- day/night/weather/road type.

Then expand in Version 2 to:

- alert time,

- full track annotations,

- impact sector,

- preventive action,

- richer context labels.

# 19. Final Recommendation

For this use case, the label schema should be:

- ego-centric,

- multi-horizon,

- actor-aware,

- localization-aware,

- explanation-ready,

- condition-aware.

The benchmark should evaluate not only whether a model predicts danger, but also:

- how early,

- for which actor,

- under what conditions,

- and with what operational false-alert cost.

That is the right enterprise-grade benchmark philosophy.

# Customer Data Requirements Checklist and Research Reading Plan

## For Ego-Centric Collision Anticipation and Driving Intelligence

# 1. Purpose

This document helps before customer data arrives.

It is designed to support:

- customer conversations,

- solution scoping,

- data request planning,

- architecture design,

- benchmark preparation,

- research study planning.

It contains two practical artifacts:

- a customer-ready data requirements checklist,

- a research reading plan aligned to ego-centric collision anticipation and related driving-intelligence use cases.

# 2. Customer-Ready Data Requirements Checklist

## 2.1 Project framing questions

These must be clarified before data transfer.

### Business objective

Ask:

- What business problem are we solving first?

- Is the first target collision prediction, near-miss detection, risky event mining, driver coaching, or real-time warning?

- Is the customer more interested in offline analytics, online warning, or both?

- What action should be triggered by the output?

### Product expectation

Ask:

- Should the system produce only event labels, or also explanation and localization?

- Does the customer want risk scoring, alerts, dashboards, or searchable event archives?

- Is the first deployment advisory-only, review-only, or safety-critical?

### Event definition

Ask:

- What counts as a collision?

- What counts as a near-collision?

- Does a crash in another lane count as a positive if ego vehicle is not involved?

- How is “dangerous driving” defined in the customer environment?

## 2.2 Video source requirements

### Camera placement

Ask for:

- mounting location,

- windshield or dashboard placement,

- field of view,

- whether all vehicles use the same setup,

- camera angle consistency across fleet,

- whether any calibration is available.

### Video format

Ask for:

- codec,

- container,

- resolution,

- FPS,

- bitrate,

- compression settings,

- audio availability,

- whether timestamps are embedded,

- whether clips are continuous or event-triggered,

- whether files contain multi-camera streams.

### Environmental conditions

Ask for expected mix of:

- day / night,

- rain / fog / glare,

- city / highway / rural,

- traffic density,

- country / road-rule differences,

- seasonal variability.

### Operational quality issues

Ask whether videos may contain:

- dropped frames,

- corruption,

- camera shake,

- dirty windshield,

- low-light noise,

- reflections,

- lens occlusion,

- mount-angle drift,

- partial blackouts.

## 2.3 Sensor and telemetry availability

Ask whether the customer can provide:

- speed,

- acceleration,

- brake events,

- steering angle,

- yaw rate,

- IMU,

- GPS,

- route / trip metadata,

- road-type metadata,

- weather metadata,

- vehicle type,

- ADAS event logs.

### Why this matters

These signals are not mandatory for a first camera-only prototype, but they can strongly improve:

- ego-motion normalization,

- time-to-collision estimates,

- behavior scoring,

- false-positive analysis,

- event explanation,

- calibration.

## 2.4 Label and annotation requirements

### Event labels

Ask whether the customer already has labels for:

- collision,

- near-collision,

- harsh braking,

- cut-in,

- unsafe lane change,

- crossing conflict,

- pedestrian conflict,

- driver coaching events.

### Temporal labels

Ask whether they can provide:

- event timestamp,

- event start and end,

- first predictable moment,

- severity level,

- reviewer confidence.

### Spatial labels

Ask whether they can provide:

- hazard actor bounding box,

- track ID,

- crash object location,

- lane region,

- impact side / direction.

### Context labels

Ask whether they can provide:

- weather,

- lighting,

- road type,

- urban/highway/intersection,

- speed regime,

- traffic density,

- visibility quality.

### Explanation labels

Ask whether they can provide cause/reason annotations such as:

- lead vehicle braking,

- cut-in from left/right,

- intersection crossing conflict,

- stopped vehicle ahead,

- side-swipe setup,

- pedestrian intrusion,

- distraction-related delayed reaction.

## 2.5 Data volume and distribution requirements

Ask for approximate counts of:

- total driving hours,

- number of vehicles,

- number of trips,

- number of collision events,

- number of near-collision events,

- normal driving hours,

- hard-negative events,

- night driving proportion,

- adverse-weather proportion.

### Important

Ask for class imbalance details because rare-event modeling is highly sensitive to positive/negative ratios.

## 2.6 Sampling and representativeness questions

Ask:

- Is the data representative of real deployment conditions?

- Are accident or risky-event clips oversampled relative to normal driving?

- Are there enough hard negatives?

- Is there distribution across different routes, cities, and vehicle types?

- Are there domain shifts such as different camera vendors or mounting angles?

## 2.7 Privacy, compliance, and governance

Ask:

- Are license plates or faces considered sensitive?

- Is audio allowed?

- What are data retention rules?

- Can raw videos leave customer systems?

- Is annotation allowed externally?

- Are there regional privacy constraints?

- Is on-prem deployment required?

- Is model training allowed on uploaded customer data?

## 2.8 Deployment requirements

Ask:

- Where will inference run: edge, cloud, hybrid?

- What is the latency requirement?

- Is online warning required?

- What edge hardware exists?

- Is GPU available in vehicle or only cloud processing?

- What bandwidth is available for uploads?

- Are clips uploaded continuously or only event-triggered?

- Is there a backend dashboard requirement from day one?

## 2.9 Evaluation and acceptance criteria

Ask:

- What matters more: recall, precision, lead time, or nuisance alert rate?

- What false alert rate is acceptable?

- What missed-event rate is acceptable?

- What review workflow is expected?

- What is the threshold for a successful pilot?

- Are there different KPIs for offline review vs online warning?

## 2.10 Customer data handoff checklist

Use this as a practical list during onboarding.

### Required minimum for Phase 1

- sample front-camera videos,

- camera specification sheet,

- event definition document,

- deployment objective (offline / online / hybrid),

- rough event counts,

- privacy and retention rules.

### Strongly recommended for Phase 1

- trip metadata,

- speed / brake / GPS logs,

- event timestamps,

- examples of true positives and false positives from existing rules if any,

- environmental diversity summary.

### Ideal for Phase 2+

- actor-level labels,

- reason labels,

- CAN/IMU,

- edge hardware details,

- reviewer workflow,

- existing dashboards or safety process docs.

# 3. Customer Discovery Questionnaire

You can ask these questions in a workshop.

## Problem and product

- What is the first business problem you want solved?

- Are you optimizing for offline analytics, online warning, or both?

- What action should happen when the system detects risk?

- What does success look like after 3 months?

## Event definitions

- How do you define collision, near-collision, and risky driving?

- Should the system only care about ego-involved events?

- What kinds of risky scenarios matter most: rear-end, cut-in, side-swipe, crossing traffic, pedestrians?

## Data and sensors

- What cameras are installed and where?

- Are videos continuous or event-triggered?

- Do you have speed, brake, GPS, steering, or IMU?

- Do you have driver-facing camera data as well?

## Labels and review

- Do you already have event labels or review notes?

- Can you provide timestamps or only clip-level labels?

- Do you have examples of false positives from prior systems?

- Who will review model outputs internally?

## Deployment and operations

- Do you need edge inference?

- What latency is acceptable?

- Can raw videos be uploaded to cloud?

- What is your privacy policy for stored clips?

- Do you need searchable event archives and dashboards?

# 4. Internal Design Checklist Before Data Arrives

Use this internally while waiting for customer data.

## 4.1 Lock down the output contract

Define now:

- event classes,

- prediction horizons,

- hazard actor output,

- impact region / sector output,

- reason code taxonomy,

- confidence and advisory levels.

## 4.2 Define the first architecture

Recommended baseline:

- ingestion,

- stream health,

- perception stack,

- object tracking,

- lane/drivable-space,

- depth,

- threat metrics,

- temporal risk model,

- fusion + alert manager,

- event clip store.

## 4.3 Define the first evaluation package

Prepare:

- research metrics,

- product metrics,

- false-alert analysis format,

- error taxonomy,

- condition-wise benchmark breakdown.

## 4.4 Define the first label schema

Prepare labels for:

- collision,

- near-collision,

- no-event,

- event time,

- hazard actor,

- reason code,

- context regime,

- visibility quality.

## 4.5 Define the first error categories

Examples:

- non-ego accident confusion,

- night glare,

- bad depth on far actors,

- tracker ID switch,

- parked-car false warning,

- cross-lane confusion,

- late alert.

# 5. Research Reading Plan

This reading plan is ordered so that you move from general understanding to highly targeted design insights.

## Stage 1 — Understand the problem space and taxonomy

### Paper 1 - Vision-Based Collision Warning Systems with Deep Learning

### Why read it

- gives the broad taxonomy of camera-based collision warning,

- distinguishes direct prediction methods from threat-metric approaches,

- highlights evaluation gaps,

- helps you think like a system designer instead of only a model builder.

### What to extract

- problem formulation taxonomy,

- architecture categories,

- deployment challenges,

- failure modes,

- metrics and limitations.

## Stage 2 — Understand ego-centric benchmark design

### Paper 2 - Nexar Dashcam Collision Prediction Dataset and Challenge (2025)

### Why read it

- highly relevant to your exact use case,

- focuses on real-world dashcam collision and near-collision prediction,

- includes event time and alert time,

- gives a practical benchmark mindset.

### What to extract

- label definition,

- event horizon design,

- prediction targets,

- evaluation metrics,

- what “good warning timing” means.

## Stage 3 — Correct your task framing using ego relevance

### Paper 3 - BADAS: Context-Aware Collision Prediction Using Real-World Dashcam Data (2025)

### Why read it

- extremely important for avoiding the wrong task framing,

- argues that many legacy datasets are not properly ego-centric,

- introduces better labeling around ego-involved events.

### What to extract

- differences between generic accident prediction and ego collision prediction,

- data annotation principles,

- benchmark pitfalls,

- recommendations for data curation.

## Stage 4 — Learn object-centric interaction modeling

### Paper 4 - Graph(Graph): A Nested Graph-Based Framework for Early Accident Anticipation (WACV 2024)

### Why read it

- shows how to model interactions among actors,

- good reference for hazard localization and multi-agent reasoning,

- useful for “with whom will ego collide?”

### What to extract

- node and edge design,

- temporal graph modeling,

- how object-centric reasoning helps over scene-only models,

- how to adapt graph logic to ego-vehicle risk.

## Stage 5 — Add 3D and depth awareness

### Paper 5 - AccNet: Real-Time Accident Anticipation for Autonomous Driving (2024)

### Why read it

- adds depth and multi-task design,

- useful for better ego-relative physical reasoning from monocular video,

- relevant for real-time deployment thinking.

### What to extract

- how depth is used,

- why 3D awareness helps,

- multi-task setup,

- real-time trade-offs.

## Stage 6 — Learn richer accident understanding and explanation

### Paper 6 - MM-AU: Abductive Ego-View Accident Video Understanding for Safe Driving Perception (CVPR 2024)

### Why read it

- useful if you want explanation, cause, and prevention understanding,

- expands beyond simple accident anticipation,

- relevant for enterprise review tools and reason-code systems.

### What to extract

- richer label schema,

- object and reason annotations,

- how accident understanding can be broader than risk prediction,

- opportunities for explainable products.

## Stage 7 — Understand classic datasets and benchmark conventions

### Paper 7 - DADA-2000

### Why read it

- classic benchmark for accident anticipation,

- useful to understand accident windows and prior literature.

### What to extract

- annotation style,

- accident object / temporal labeling,

- limitations vs newer ego-centric benchmarks.

### Paper 8 - DoTA

### Why read it

- very useful for “when / where / what” anomaly understanding,

- strong reference for spatial-temporal event labeling.

### What to extract

- localization task design,

- anomaly timeline structure,

- metric style.

## Stage 8 — Extend toward driver behavior and action understanding

### Paper 9 - DriveAction (2025)

### Why read it

- directly relevant if you later expand into driver behavior or maneuver understanding,

- connects egocentric driving video to action and decision labels.

### What to extract

- driving action taxonomy,

- maneuver understanding formulation,

- how this can share infrastructure with collision anticipation.